AUSTROADS TEST METHOD ATM 030

Calculation of the Characteristic Value of a Lot

Contents

1.	Preface	1

2.	Scope	1

3.	References	1

4.	General	2

5.	Procedure	2

6.	Calculation	3

7.	Test Report	4

Amendment Record	6

# Preface

Where a statistical approach to product acceptance is used, the characteristic value for the lot is determined by analysing multiple random test results from the lot.

Characteristic values may be specified for properties such as geotextile strength, compaction of pavement and earthworks layers and the voids ratio of asphalt.

# Scope

This method provides a means for calculating a characteristic value determined by the analysis of multiple individual test results, tested using the same methods, using a statistical process.

# References

Nil

# General

Where an Austroads Technical Specification (ATS) specifies a characteristic value for acceptance, the characteristic value shall be determined in accordance with the procedure in Section 5.

For the purpose of this test method, a test result includes a single measurement of an attribute.

This test method only applies where the test results follow a normal distribution curve.

# Procedure

The procedure shall be as follows:

- Perform the required number of tests specified in the ATS in accordance with the specified test method.

Calculate the mean and standard deviation from the individual test results in accordance with Section 6.

Determine the acceptance constant (k) as follows:

if a k value is specified in the applicable ATS, use that value; otherwise,

determine the k value from Table 7.1 (which is based on the number of tests carried out and the assumed “producer’s risk” of 10%).

# Calculation

- Calculate the mean of the individual test results as follows:

|  |  | x= 1ni=1nxi |  |
| --- | --- | --- | --- |

Calculate the standard deviation of the individual test results as follows:

|  |  | s=n=1nxi-x2n-1 |  |
| --- | --- | --- | --- |

Calculate the characteristic value using the appropriate method as follows:

|  |  | QL = x – ks (for a minimum limit) QU = x+ks (for a maximum limit) |  |
| --- | --- | --- | --- |
| where |  |  |  |
| n | = | Number of test results |  |
| xi | = | Individual test result for i = 1, 2, 3… n |  |
| x | = | The mean of the individual test results for i =1, 2, 3… n |  |
| s | = | Standard deviation of the individual test results for i = 1, 2, 3… n |  |
| k | = | Acceptance constant |  |
| QL | = | Characteristic value of attribute (lower or minimum limit) |  |
| QU | = | Characteristic value of attribute (upper or maximum limit) |  |

# Test Report

In addition to the reporting specified for the applicable test method, the following shall be reported:

- Report the following values rounded to a number of significant figures which exceeds by one that normally reported for the individual test results:

mean of the individual test results, and

standard deviation of the individual test results.

Report the characteristic value of the individual test results to the same rounding normally applied to individual test results.

Report the following additional values:

number of individual tests, and

acceptance constant (k) used to the nearest 0.001.

Report the following additional information:

source of the acceptance constant (k) used; for example, Table 7.1

identification of the specification requiring the reporting of the characteristic value of the individual test results

statement identifying use of unrounded data in calculation of characteristic value, and

the number of this test method, that is ATM 030.

Table 7.1	Acceptance constants

| Number of tests or measurements | Acceptance constant (k) |  | Number of tests or measurements | Acceptance constant (k) |
| --- | --- | --- | --- | --- |
| 2 | 0.403 |  | 19 | 0.937 |
| 3 | 0.535 |  | 20 | 0.946 |
| 4 | 0.617 |  | 21 | 0.952 |
| 5 | 0.675 |  | 22 | 0.959 |
| 6 | 0.719 |  | 23 | 0.965 |
| 7 | 0.755 |  | 24 | 0.972 |
| 8 | 0.783 |  | 25 | 0.978 |
| 9 | 0.808 |  | 30 | 1.002 |
| 10 | 0.828 |  | 35 | 1.020 |
| 11 | 0.847 |  | 40 | 1.036 |
| 12 | 0.863 |  | 45 | 1.049 |
| 13 | 0.877 |  | 50 | 1.059 |
| 14 | 0.890 |  | 60 | 1.077 |
| 15 | 0.901 |  | 70 | 1.091 |
| 16 | 0.910 |  | 80 | 1.103 |
| 17 | 0.919 |  | 90 | 1.112 |
| 18 | 0.928 |  | 100 | 1.120 |

Notes:

- Table 7.1 is based on based on a ‘producer's risk’ of 10%; i.e., a probability of 10% that work which is actually conforming is rejected. If an alternative ‘producer’s risk’ is required, other values of k will be appropriate; for example, for a producer’s risk of 20%, the value of k for 6 test results is 0.92.

- Some ATS may require that the value of k for 10 test results in Table 7.1 is used, even though less than 10 test results are available. This may be referred to as a ‘truncated statistical acceptance scheme’.

Amendment Record

| Amendment no. | Clauses amended | Action | Date |
| --- | --- | --- | --- |
| – | New test method | New | February 2025 |

| Key |  |
| --- | --- |
| Format | Change in format |
| Substitution | Old clause removed and replaced with new clause |
| New | Insertion of new clause |
| Removed | Old clauses removed |

Acknowledgement This Austroads Test Method includes content from:

Queensland Transport and Main Roads: Test Method Q020: Calculation of characteristic value of a lot. Transport for NSW: Specification Q6 Quality Management System Annexure L. Main Roads WA: Specification 201 Quality Management Annexure 201A. Department of Transport and Planning Victoria: Test Method RC 316.00. Density Ratio and Moisture Ratio - Lot Characteristics.