| AUSTROADS TECHNICAL SPECIFICATION ATS 4220 Wire Rope Safety Barrier |  |
| --- | --- |
| Contents 1. Scope 1 2. Referenced Documents 2 3. Definitions 3 4. Quality System Requirements 3 5. Materials and Components 4 General 4 Protective Treatment 4 Identification 4 Material Handling and Storage 5 Certification of Conformance 5 6. Personnel 5 7. Installation 6 General 6 Set Out 6 Post Foundations and Anchor Blocks 7 Posts 8 Loading Test of Post Foundations 8 Location of Posts Adjacent to Batter Hinge Points 8 Wire Rope 9 WRSB over Box Culverts and Concrete Pipes 9 Stray Current Effects on WRSB 9 Interface with Other Safety Barriers 10 Delineators 10 8. Removal of Existing Roadside Furniture 10 9. Damage and Reinstatement 11 10. Tolerances 11 11. Completion 12 Annexure A Summary of Hold Points, Witness Points and Records 13 Amendment Record 14. |

# Scope

Austroads Technical Specification ATS 4220 sets out the requirements for the supply and installation of wire rope safety barrier (WRSB) systems.

ATS 4220 may be used for new installations and for the repair or replacement of existing WRSB systems.

The following are excluded from this Specification:

design of the WRSB system;

steel rail safety barrier systems (refer to ATS 4210);

concrete safety barriers (refer to ATS 4230);

temporary safety barriers used during the construction of roadworks or bridgeworks; and

balustrades, pedestrian fences or other fences adjacent to roads.

# Referenced Documents

The following documents are referenced in this Specification:

Australian/New Zealand Standards AS/NZS 1214 Hot-dip galvanized coatings on threaded fasteners (ISO metric coarse thread series) (ISO 10684:2004, MOD) AS 1742.2 Manual of uniform traffic control devices, Part 2: Traffic control devices for general use AS 1906 Retroreflective materials and devices for road traffic control purposes, Part 1: Retroreflective sheeting AS/NZS 1906 Retroreflective materials and devices for road traffic control purposes, Part 2: Retroreflective devices (non-pavement application) AS 2700 Colour standards for general purposes AS/NZS 3750 Paints for steel structures AS 4506 Metal finishing – Thermoset powder coatings AS/NZS 3000 Electrical installations (known as the Australian/New Zealand Wiring Rules) AS/NZS 4680 Hot dip galvanized coatings on fabricated iron and steel articles – Specifications and test methods AS/NZS ISO 9001 Quality management systems – Requirements

Austroads AGRD06 Guide to Road Design Part 6: Roadside Design, Safety and Barriers ATS 1210 Temporary Traffic Management ATS 1330 Management of Utilities on Site ATS 4210 Steel Beam Safety Barrier ATS 4230 Construction of Concrete Safety Barriers ATS 5330 Supply of Geopolymer Concrete ATS 5335 Normal Class Concrete

# Definitions

In addition to the definitions in Appendix A of AGRD06, the following definitions and abbreviations apply to this Specification.

| Accepted WRSB System: | A WRSB System that has been approved by the Principal and is subject to compliance with any conditions of use specified by the Principal. |
| --- | --- |
| ASHTAS: | Austroads Safety Hardware Training and Accreditation Scheme. |
| Design Documentation | Drawings, schedules and other documents, designated as such, required for the installation of the safety barrier system, including the Manufacturer’s Recommendations. |
| Manufacturer’s Recommendations: | The approved specification, instructions, installation manual and/or drawings for an Accepted WRSB System, prepared by or for the manufacturer, detailing the components, the system or device, and the methods and/or procedures for installation. |

# Quality System Requirements

The Contractor must prepare and implement a Quality Plan that includes the documentation in Table ‎4.1.

Table ‎4.1 Quality Plan

| Clause | Description of document |
| --- | --- |
| ‎4.2 | - Evidence that the WRSB System is an Accepted WRSB System. |
| ‎7.1 | - Procedures and Inspection and Test Plans for verifying that the WRSB is installed in accordance with the Manufacturer’s Recommendations and this Specification. |

An Accepted WRSB System must be used for all new WRSB installations. Acceptance may be conditional upon the WRSB complying with specific requirements specified by the Principal. These specific requirements apply to the use of an Accepted WRSB System in accordance with this Specification.

All components of the WRSB system must be manufactured and supplied under a quality system certified to AS/NZS ISO 9001. The certifying body must be accredited by the Joint Accreditation System of Australia and New Zealand (JAS-ANZ) or a member of the International Accreditation Forum.

| HOLD POINT 1 |  |
| --- | --- |
| Process Held | Installation of the WRSB system |
| Submission Details | The Quality Plan must be provided to the Principal at least 10 working days prior to the installation of the WRSB system. |

# Materials and Components

## General

Materials and components used in the WRSB must comply with the Manufacturer’s Recommendations for the Accepted WRSB System and satisfy the requirements of the system for a minimum design life of 20 years.

Materials other than steel that are used in the manufacture or installation of WRSB must comply with Table ‎5.2 and the requirements specified on the Design Documentation.

Table ‎5.2 Other materials

| Item | Compliance requirement |
| --- | --- |
| Retroreflective materials and combination of fluorescent/retroreflective materials used in delineators | AS 1906.1 or AS/NZS 1906.2, as appropriate |
| Concrete | Strength Grade as specified: Normal class concrete to ATS 5335; or Geopolymer binder-based concrete to ATS 5330. |

## Protective Treatment

Steel posts must be either:

hot dipped galvanised in accordance with AS/NZS 4680;

pre-galvanised in accordance with Z600 coating type to AS 1397 (total minimum coating mass on both surfaces of 600 g/m2); or

pre-galvanised in accordance with ZM275 coating type to AS 1397 (total minimum coating mass on both surfaces of 275 g/m2).

The weld seam made during forming of pre-galvanised steel post sections must be repaired in accordance with Section 3 in AS/NZS 4792 by applying a coating using a metal spraying system and employing a suitable zinc or zinc-alloy wire/powder.

All galvanised coatings must be smooth, adherent and free from stains, gross surface imperfections, markings, brand names and/or inclusions.

Unless specified in the Contract documents that a powder coating is not required, the hot dipped galvanised posts or pre-galvanised posts must be powder coated with a thermosetting polyester powder coating complying with AS 4506. The powder coating must provide a grainy, low gloss appearance. Except where green posts are specified, the coating must be white (colour G14 to AS 2700). If green posts are specified, the colour must be G61 to AS 2700.

The hot-dipped galvanised coating on bolts, nuts and washers must comply with AS/NZS 1214.

Refer to Clause ‎9 if the coating is damaged.

## Identification

All posts must be permanently and clearly marked with the manufacturer’s identification mark, which must be clearly visible at the completion of the installation. The marking must be in text that is not more than 20 mm high and be in an unobtrusive and readily accessible position.

## Material Handling and Storage

The Contractor must ensure that loading, transport, unloading, stacking and handling operations, before and after fabrication, are carried out in such a way that items are protected from distortion and that galvanised surfaces are protected from damage.

All materials and components must be stored in such a manner that damage and corrosion are prevented. Generally, storage at least 200 mm above the ground on platforms, slabs, or other supports under cover will be satisfactory. Rusted, bent or damaged steel must not be incorporated into the Works.

Wire rope must be supplied on reels and must not be twisted or kinked.

Components must be stored in such a manner that the freshly galvanised surfaces are protected from the attack of ‘white rust’. The components must not be transported or stored under damp or badly ventilated conditions (including stacking components in close contact with each other).

If stacks are located behind a serviceable road safety barrier system, the clear space between the road safety barrier system and the stack must allow for the Working Width of the system and be at sufficient distance from the ends of the system to allow the proper functioning of the end treatments.

## Certification of Conformance

At least 10 working days prior to the installation of the WRSB components, the Contractor must submit a signed statement certifying that the materials and components supplied comply with the requirements of this Specification. The statement must be signed by a person authorised by the Contractor to sign on its behalf. The certificate must be accompanied by:

a document identifying all relevant national or international standards with which the WRSB system and its components comply with;

the manufacturer’s name, product reference and date of manufacture;

details of the country of manufacture of each component;

the steel manufacturer’s test certificates showing full chemical composition, including the carbon equivalence, and mechanical test results for yield strength, tensile strength and elongation;

test certificates for any welding;

for galvanised steel components, a manufacturer’s certificate of compliance certifying that the zinc coating mass is in accordance with the requirements of AS/NZS 4680 or AS 1397 (as appropriate); and

test certificates for powder coating (where applicable).

| HOLD POINT 2 |  |
| --- | --- |
| Process Held | Installation of the WRSB system |
| Submission Details | The certificate of compliance and supporting documentation must be provided to the Principal at least 10 working days prior to the commencement of installation. |

# Personnel

The installation, repair or maintenance of a safety barrier system must be undertaken by a person who is accredited under the ASHTAS for the safety barrier system that is being installed, repaired or maintained.

# Installation

## General

The Quality Plan must include the following documentation:

- details of the equipment used to install the WRSB components;

- procedure for installation of the WRSB components; and

- the Inspection and Test Plan, to verify that the WRSB is installed in accordance with this Specification.

The WRSB system must be installed in accordance with the Design Documentation and any Principal’s conditions of use attached to the acceptance of a product.

Prior to commencing the installation of the WRSB system, the Contractor must ensure that the position of all Utilities in the proximity of the Works has been verified and any necessary protective measures implemented in accordance with ATS 1330. If an unknown Utility is encountered during the installation process, the requirements of ATS 1330 will apply.

The installation of the WRSB system must be undertaken in accordance with the Traffic Management Plan (refer to ATS 1210).

Where the WRSB system is being constructed on a road open to traffic, the work must commence at the end closest to approaching traffic, except where the barrier connects at its departure end to a fixed object such as an existing barrier.

The work must be carried out so end treatments and transitions can be commissioned at the earliest practicable time. Where this cannot be achieved on the same day that the remainder of the WRSB system is installed, temporary end treatments in accordance with the Traffic Management Plan must be provided until the permanent treatment is complete.

## Set Out

Prior to installation, the Contractor must set out the alignment of the posts and anchor blocks using pegs or other suitable marking system. The Contractor must verify that:

there are no impediments on the Site that impede the construction of the WRSB in the correct position; and

the position of the posts comply with all specified requirements, such as offset from the batter hinge point (as described in AGRD06) and the minimum clearance required for the Working Width.

If a previously unknown impediment to installation or a non-compliance with a specified requirement is identified, the Principal must be notified as soon as practicable and before post installation commences.

| WITNESS POINT 1 |  |
| --- | --- |
| Process | Installation of posts |
| Notification Period | At least 2 working days prior to the commencement of the post installation. |

## Post Foundations and Anchor Blocks

Where a geotechnical investigation has not been undertaken, the nominated default anchor and post footing must be used in accordance with the Manufacturer’s Recommendations.

If any of the following conditions apply, the Contractor must ensure that a geotechnical investigation is undertaken to determine the soil properties:

the Contractor proposes to use an alternative anchor or post foundation to that identified within the Manufacturer’s Recommendations; or

site constraints restrict the use of the default or an approved alternative anchor or post foundation.

If Clause ‎7.10 b) applies, the Contractor must provide the geotechnical information to the WRSB supplier and obtain a written statement from the supplier that the anchor block size and post foundation size are appropriate for the existing natural or constructed ground conditions and the installation meets the operational requirements.

The geotechnical investigation must verify that the completed in situ concrete post foundation system meets the requirements of the Accepted WRSB System and will not crack, lift or displace during impact at the nominated test level during the design life of the barrier.

At a minimum, the geotechnical investigation must provide values of the following:

soil bearing pressure;

soil type; and

average allowable soil stress.

The Contractor must include sufficient time for the geotechnical investigation in its program of work.

| HOLD POINT 3 |  |
| --- | --- |
| Process Held | Installation of non-standard post foundations and anchor blocks |
| Submission Details | The geotechnical investigation and the corresponding Design Documentation must be submitted to the Principal at least 5 working days prior to the installation of the post foundations and anchor blocks. |

Anchor block foundation and post foundation holes must be free of loose material, debris and water prior to the placement of concrete.

Any over-excavation of the anchor block must be cleaned out and filled with concrete forming the concrete anchor block. Reforming the excavation with formwork or soil is not permitted.

| WITNESS POINT 2 |  |
| --- | --- |
| Process | Placement of concrete for the post foundations and anchor blocks |
| Notification Period | At least 2 working days before the concrete pour. |

The minimum strength of concrete (refer Table ‎5.2) must comply with the following:

In situ concrete post foundations:	25 MPa

Anchor blocks:			32 MPa

All anchor frames, posts, sockets and reinforcing rings must be positioned to the line and levels as specified in the Design Documentation and must be secured against displacement during placing of the concrete.

Construction joints within the anchor block are not permitted.

The finished surface of all anchor blocks and footings must be shaped such that water cannot pool on the surface.

## Posts

The Contractor must install the wire rope and posts to within the tolerances specified in Table ‎10.1.

Posts must be installed with their correct profile in the direction of travel.

The length of the post socket or overall length of the post may be adjusted in accordance with the tolerances specified by the manufacturer to achieve the specified vertical alignment. Any such adjustment must be in accordance with the Manufacturer’s Recommendations. The Contractor must obtain written confirmation from the manufacturer that the adjustment is acceptable and submit the confirmation to the Principal.

## Loading Test of Post Foundations

Horizontal load testing of posts with concrete foundations must be undertaken prior to installation of the wire rope if directed by the Principal or if any of the following applies:

non-standard post foundations are to be used, i.e. not the manufacturer’s nominated default foundation;

the soil type cannot be verified;

ground conditions are such that the required depth of footing cannot be installed;

adverse geological conditions, i.e. a rock cut section where the post foundation may be reduced in length, or where soft clay or medium to loose sand profiles have been identified and the standard foundation dimensions may be insufficient; or

Clause ‎7.29 applies.

Horizontal load testing of posts must be carried out by applying a force of 10 kN to the post 600 mm above ground level at an angle of 45 degrees to the WRSB. The post foundation is deemed non-conforming if the displacement at ground level exceeds 3 mm.

The test may be carried out by placing a post into the completed concrete footing and using a lever hoist or equivalent and calibrated load cell anchored to a truck.

At the completion of the load testing, the Contractor must remove the test posts and rectify any disturbed or damaged post foundations.

The result of all tests must be photographed and logged against location. For any footings failing the test, relevant advice must be sought from the manufacturer. Remedial measures and re-testing of the rectified post plus one other similar post must be undertaken.

## Location of Posts Adjacent to Batter Hinge Points

If the specified position of a post is less than the Working Width from the batter hinge point (as described in AGRD06), the Contractor must:

increase the depth of footing in accordance with the Manufacturer’s Recommendations;

provide a written statement of advice from the manufacturer based on geotechnical testing confirming the required depth of footing; and

undertake horizontal load testing in accordance with Clause ‎7.25 at 45 degrees to the WRSB in all directions at locations nominated by the Principal.

## Wire Rope

The vertical alignment of the wire ropes must be smooth and uniform, without sudden changes in gradient and generally consistent with the vertical alignment of the edge of the traffic lanes.

In the event that the horizontal and/or vertical geometry of the alignment do not satisfy the Manufacturer’s Recommendations or this Specification, the Contractor must immediately notify the Principal and provide details of the deficient areas along with the proposed dispositions. Any adjustments accepted by the Principal must then be made to the alignment or the level of the median or outer verge to ensure that the installation complies with the requirements of the Manufacturer’s Recommendations and this Specification.

Initial tensioning of each rope must not proceed until at least 7 days after pouring the last anchor block or post footing that will be affected by tension in the ropes.

Each wire rope must be tensioned in accordance with the relationship between rope tension and ambient temperature, applicable to the WRSB system in accordance with the Manufacturer’s Recommendations. The tension must be measured using a tension meter supported with a current calibration certificate.

Prior to commissioning the WRSB system, but not less than 3 months after the initial tensioning of the WRSB, the tension in each wire rope must be checked, and re-tensioned if required, to ensure compliance with the ambient temperature/rope tension relationship.

| WITNESS POINT 3 |  |
| --- | --- |
| Process | Checking, and re-tensioning if required, of tension in each wire rope |
| Notification Period | At least 2 working days before the concrete pour. |

The Contractor must prepare a tensioning report and submit the report to the Principal within 5 working days of the tensioning works being carried out.

## WRSB over Box Culverts and Concrete Pipes

Where WRSB straddles box culverts or a concrete stormwater pipe, the adjacent posts must be located equidistant from the centre of the culvert or pipe. The spacing of other posts in the WRSB may be adjusted locally to meet this requirement.

## Stray Current Effects on WRSB

Unless directed otherwise by the Principal, all wires within a WRSB system must be tested for induced currents and voltages relative to all other adjacent wires and the ground using calibrated equipment in accordance with AS/NZS 3000 by appropriately qualified and licensed personnel. Tests must be undertaken at each location where 2 sections of WRSB overlap at anchorage points. Where an induced current or voltage is recorded, or high voltage powerlines are within 50 m of the barrier, the WRSB system must be earthed to remove current or voltage running through the wires.

All wires in the WRSB and overlapping sections of WRSB must be connected to earth using a flexible system. The earthing must occur at the location of overlapping WRSB and at a maximum of 500 m intervals along the length of each affected section of WRSB. Detailed of the earthing must be submitted to the Principal.

Prior to putting the WRSB system into service, the earthing must be checked to ensure that any further stray current effects have been eliminated.

## Interface with Other Safety Barriers

Unless specified otherwise in the Contract documents, the Contractor is responsible for any alteration and/or relocation of any adjacent existing safety barrier system to ensure the proper functioning of the safety barrier system. This may include dismantling and reinstating existing safety barrier (including terminals) and any minor site works. If a steel beam safety barrier requires alteration and/or relocation, that work must be carried out in accordance with ATS 4210.

New WRSB must not be connected to existing WRSB unless it is the same system comprising the same components.

## Delineators

The Contractor must supply and fasten delineators to the WRSB system, comprising post caps fitted with 50 mm × 100 mm Class 1A retro-reflective material, as defined in AS/NZS 1906.2, or placed on the front top of the post above the wire ropes. Delineators must be installed at the spacing specified in AS 1742.2 or the Design Documentation, whichever gives the closer spacing. Delineators must not be placed on flared sections or on sloped terminals.

Delineators must be installed to the top of the WRSB system in accordance with any applicable Design Documentation. The delineators must be positioned so that vehicles approaching from either direction at night will only see:

red delineators on the left side of one-way and two-way roadways;

white delineators on the right side of two-way roadways; and

yellow delineators on the right side of one-way roadways and medians separating traffic in opposing directions.

Delineators must not be installed on a WRSB when the barrier offset is greater than 4 m from the traffic lane.

# Removal of Existing Roadside Furniture

Existing roadside furniture (including safety barriers and guideposts) must be removed where shown on the Design Documentation. If any of the following scenarios apply and removal is not shown on the Design Documentation, the Contractor must notify the Principal accordingly and seek a direction in regard to their removal:

the roadside furniture will be made redundant by the installation of the WRSB;

the roadside furniture is located within the Working Width of the WRSB at the nominated test level;

a guidepost is located in front of a new WRSB within a 4 m offset from the traffic lane; or

a guidepost is located behind the new WRSB and is greater than 4 m offset from the traffic lane.

Where the safety barrier system being removed is on a road open to traffic, the removal must be coordinated with other work at the Site to eliminate or minimise the exposure of an incomplete safety barrier system to traffic. Where practicable, removal of a safety barrier system must commence from the departure end (remote from the approach of traffic).

Unless specified otherwise in the Contract documents, the following applies to the demolition of any existing safety barrier or other roadside furniture:

all components of the redundant safety barrier and any waste materials must be removed from the site in accordance with the environmental management requirements included in the Contract documents;

any other roadside furniture (such as guide posts or bollards) that has been made redundant by the installation of the safety barrier, must be removed from the site;

any holes remaining after the removal of the redundant roadside furniture must be cleaned and backfilled with similar materials to existing surrounding layers or imported granular material; and

the backfilled material must be compacted in layers not exceeding 150 mm thickness to not less than the density of the surrounding material.

# Damage and Reinstatement

Components of the WRSB system must not be damaged at any stage during supply, fabrication or installation. However, if the damage is limited to the protective coating, repair in accordance with Appendix E of AS/NZS 4680 is acceptable, subject to the following:

if practicable, the damage is repaired immediately, but in any case, within 24 hours;

the damaged area does not exceed 40 cm2 for any individual point repaired and a total 0.1% of the surface area of any face (for multiple repairs);

at least  2 coats of a zinc-rich paint in accordance with AS/NZS 3750.9 are used; and

the colour of the original surface is matched by either directly or by applying a further compatible treatment.

If the coating cannot be repaired in accordance with Clause ‎9.1, the component must be either replaced with a conforming component or re-galvanised in accordance with this Specification.

If, during the period that the Contractor is responsible for the care and control of the Works, the WRSB is damaged by an errant vehicle, the Contractor must:

- implement traffic management measures and make the area safe as soon as practicable after becoming aware of the damage;

- repair the damage within 5 working days of becoming aware of impact damage to the WRSB; and

- provide a report to the Principal of each vehicle impact where the WRSB has been damaged.

Any property of the Principal (such as kerb and channel, footpaths, verges, signs and traffic control devices) that is damaged or temporarily removed/relocated during the execution of the work must be reinstated to its original location as soon as practicable and to a standard not less than the pre-existing condition.

# Tolerances

The WRSB must be installed within the tolerances specified in Table ‎10.1. In addition, the WRSB must be installed within any tolerances specified by the manufacturer.

Table ‎10.1 Installation tolerance

| Position or dimension | Tolerance |
| --- | --- |
| Height of the WRSB | ±20 mm from the design line |
| Longitudinal line | ±20 mm in plan view |
| Post spacing – distance from specified position | ±25 mm |
| Deviation of the post from the upright axis | ±15 mm at the top of the barrier |

WRSB height must be measured from the road pavement when it is located within 1.5 m of the edge of pavement. For distances beyond 1.5 m, the WRSB height must be measured from the ground surface at its location.

# Completion

## General

As a condition precedent for payment and for Completion or Practical Completion (as the case may be), the following must be submitted to the Principal:

certification of materials and supporting documentation required under Clause ‎5.15;

a copy of each completed Inspection and Test Plan and photographic records demonstrating compliance with Clause ‎7;

a Certificate of Compliance and supporting documentation required under Clause ‎11.3; and

the ‘As-constructed’ and maintenance documentation under Clause ‎11.5.

## Certificate of Compliance

The Contractor must arrange for a compliance audit of each WRSB system constructed under the Contract. The audit must be undertaken and a report prepared by the approved Australasian supplier of the WRSB system.

A Certificate of Compliance, signed by the Contractor’s Representative and the supplier of the Accepted WRSB System, and all documentation supporting that certification must be submitted to the Principal. The Certificate of Compliance must certify that the WRSB system has been installed in accordance with the Manufacturer’s Recommendations, this Specification and the Principal’s conditions of use (where applicable).

A person who undertakes the installation, repair and/or maintenance of WRSB systems and is licensed under the ASHTAS may certify that the work is conforming work in lieu of a Certificate of Compliance. Any certification of installation, repair, or maintenance work must correspond to the WRSB system for which the individual is accredited.

## ‘As-constructed’ and Maintenance Documentation

The Contractor must submit the following documentation to the Principal:

- installation and maintenance manuals for each WRSB System installed;

- ‘As-constructed’ drawings; and

- any other information required by the Principal for inclusion in its asset management system.

Summary of Hold Points, Witness Points and Records

The following is a summary of the Witness Points/Hold Points that apply to this Specification and the Records that the Contractor must submit to the Principal to demonstrate compliance with this Specification.

| CLAUSE | HOLD POINT | WITNESS POINT | RECORD |
| --- | --- | --- | --- |
| ‎4.1 |  |  | Quality Plan |
| ‎4.3 | - Installation of WRSB system |  |  |
| ‎5.15 | - Installation of WRSB system |  | The certificate of compliance for the materials and supporting documentation |
| ‎7.8 |  | - Installation of posts |  |
| ‎7.14 | - Installation of non-standard post foundations and anchor blocks |  | Geotechnical investigation and the corresponding Design Documentation |
| ‎7.16 |  | - Placement of concrete for the post foundations and anchor blocks |  |
| ‎7.34 |  | - Checking, and re-tensioning if required, of tension in each wire rope |  |
| ‎11.1 |  |  | Evidence of compliance |

Amendment Record

| Edition no. | Clauses amended | Action | Date |
| --- | --- | --- | --- |
| 1.0 | New specification | New | January 2026 |

| Key |  |
| --- | --- |
| Format | Change in format |
| Substitution | Old clause removed and replaced with new clause |
| New | Insertion of new clause |
| Removed | Old clauses removed |