| AUSTROADS TECHNICAL SPECIFICATION ATS3110 Supply of Polymer Modified Binders |  |
| --- | --- |

Contents 1. Scope 1 2. Definitions 1 3. Referenced Documents 2 4. Quality System Requirements 3 5. Manufacture of Binders 3 6. Sampling and Testing of Binders 3 7. Records 4 8. Properties of Binders 4 9. Frequency of Testing 7 Annexure A: Summary of Hold Points, Witness Points and Records 9

# Scope

This Austroads Technical Specification ATS 3110 sets out the requirements for the supply of polymer modified binders (PMBs) and crumb rubber modified binders for use in both sprayed sealing and asphalt applications.

# Definitions

The following definitions apply to this Specification:

Field-produced crumb rubber modified binders: 
Those binders that are simple blends of bitumen and crumb rubber which are blended close to the application site and are used within a short time frame (in no case more than 2 hours travelling time from the blend site).

Batch: 	The quantity of polymer modified binder stored in a single tank by the manufacturer at any particular time. The binder in the storage tank is deemed to be a new batch whenever new material is added to the storage tank.

The definitions contained in the following documents:

AP-C87-15 Austroads Glossary of Terms

Standards and guidelines referenced in this Specification.

Refer to the Austroads Guide to Pavement Technology Part 4F: Bituminous Binders for details of the classification system applicable to PMBs.

# Referenced Documents

The following documents are referenced in this Specification or are relevant to this Specification:

Australian /New Zealand Standards

AS/NZS 2341.4: 	Methods of testing bitumen and related roadmaking products: Method 4: Determination of dynamic viscosity by rotational viscometer.

AS/NZS/ISO 9001: 	Quality Management Systems: Requirements, International Organisation for Standardisation

Austroads Test Methods (available from: https://austroads.com.au/publications)

AGPT/T101	Method of sampling polymer modified binders, polymers and crumb rubber

AGPT/T102 	Protocol for handling modified binders in preparation for laboratory testing

AGPT/T103 	Pre-treatment and loss on heating of bitumen, multigrade and polymer modified binders (rolling thin film oven (RTFO) test)

AGPT/T108 	Segregation of polymer modified binders

AGPT/T111 	Handling viscosity of polymer modified binders (Brookfield Thermosel)

AGPT/T112 	Flash point of polymer modified binders

AGPT/T121	Shear properties of polymer modified binders (ARRB elastometer)

AGPT/T122 	Torsional recovery of polymer modified binders

AGPT/T125	Stress ratio of polymer modified binders using the Dynamic Shear Rheometer (DSR)

AGPT/T131 	Softening point of polymer modified binders

AGPT/T132 	Compressive limit of polymer modified binders

AGPT/T142 	Rubber content of digested crumb rubber binders – trichlor bath method

AGPT/T143 	Particle size and properties of crumb rubber

AGPT/T144 	Morphology of crumb rubber – bulk density test

Austroads (available from: https://austroads.com.au/publications)

AGPT04F-17	Guide to Pavement Technology Part 4F: Bituminous Binders

AGPT04K-18	Guide to Pavement Technology Part 4K: Selection and Design of Sprayed Seal

AGPT08-09	Guide to Pavement Technology: Part 8: Pavement Construction.

AP-G41-15	Bituminous Materials Safety Guide

Australian Asphalt Pavement Association (AAPA)

Advisory Note 7	Guide to the heating and storage of binders for sprayed sealing and asphalt manufacture.

# Quality System Requirements

The PMB must be manufactured under a quality management system which is certified to AS/NZS ISO 9001 by a JASANZ accredited certifier (or accredited by another Accreditation Body Member of the International Accreditation Forum).

# Manufacture of Binders

The manufacturer must implement a documented process control system to produce PMBs of a consistent quality conforming to the requirements of this Specification.

As a minimum, the process controls must include:

- a method for determining and controlling the formulation during the production process;

keeping records of the composition of the constituent materials for each batch; and

recording sampling frequencies and test results.

The manufacturer must:

- operate an Inspection and Test Plan (ITP) which demonstrates that the PMB complies with this specification and includes testing of the PMB, analysis of results (including control charts);

ensure the all PMB supplied can be traced to the production batch and associated test report; and

ensure that procedures / guidelines for the handling, storing, and transport of the binders that ensures homogeneity and conformity at the time of incorporation into the works are readily available to the Principal and Contractor.

# Sampling and Testing of Binders

Representative samples of PMBs must be obtained in accordance with AGPT/T101. PMB samples must be prepared for testing using AGPT/T102.

Binders must be tested in accordance with the Test Methods specified in Tables 8.1, 8.2 and 8.4.

The minimum frequency of testing must be in accordance with Tables 9.1 and 9.2

Unless specified otherwise, the properties listed in Tables 8.1 to 8.4 refer to testing of samples taken at the point of manufacture.

Sampling must be undertaken by a person who is competent in that sampling procedure. If requested, the Contractor must provide training records demonstrating evidence of that person’s competency.

The manufacturer must nominate in its Quality Management System the tests to be carried out prior to batch release of factory blended binders. At a minimum, this must include the requirements set out in Table 8.1 and 8.2.

# Records

The Contractor must ensure that the following records are made available prior to, or with, the delivery of the PMB:

- Test results, from a NATA accredited laboratory, demonstrating that the PMB has been sampled and tested in accordance with Tables 8.1 to 8.4 and complies with the properties specified in those tables.

A delivery docket giving at least the following information:

name and address of the manufacturer;

location and date of manufacture;

polymer modified binder class;

production batch number;

storage and heating information (i.e. location, date, time, temperature; and

certification that the PMB has been sampled prior to release from the manufacturer and complies with this Specification.

# Properties of Binders

The PMB properties must comply with the with the values specified in the following tables:

| Sealing Class Binders: | Table 8.1 |
| --- | --- |
| Asphalt Class Binders: | Table 8.2. |
| Field-Produced Crumb Rubber Modified Binders: | Table 8.3 |

Table 8.1:	Properties of Polymer Modified Binders for Sprayed Sealing Applications

| Test method | Class Binder property | S10E | S15E | S20E | S25E | S35E | S45R(1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AS/NZS 2341.4 or AGPT/T111(2) | Viscosity at 165 °C (Pa.s) max.(2) | 0.55 | 0.55 | 0.6 | 0.9 | 0.55 | 4.5(2) |
| AGPT/T122 | Torsional recovery at 25 °C, 30 s (%) | 22–50 | 32–62 | 38–70 | 55–80 | 16–32 | 25–55 |
| AGPT/T131 | Softening point (°C) | 48–64 | 55–75 | 65–95 | 82–105 | 48–56 | 55–65 |
| AGPT/T125 | Stress ratio at 10 °C min. | TBR(3) | TBR | TBR | TBR | TBR | TBR |
| AGPT/T121 | Consistency 6% at 60 °C (Pa.s) min.(4) | 300 | 400 | 500 | 900 | 250 | 800 |
| AGPT/T121 | Stiffness at 15 °C (kPa) max. | 140 | 140 | NA(5) | NA | 180 | 180 |
| AGPT/T121 | Stiffness at 25 °C (kPa) max. | NA | NA | 35 | 30 | NA | NA |
| AGPT/T132 | Compressive limit at 70 °C, 2 kg (mm) min. | NA | NA | NA | NA | NA | 0.2 |
| AGPT/T108 | Segregation (%) max. | 8 | 8 | 8 | 8 | 8 | 8 |
| AGPT/T112 | Flash point (°C) min. | 250 | 250 | 250 | 250 | 250 | 250 |
| AGPT/T103 | Loss on heating (% mass) max. | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |

Notes

Class S45R binder must be manufactured by the incorporation of crumb rubber derived from used vehicle tyres.

L series Brookfield is recommended together with spindle SC4-31, except in the case of S45R where spindle SC4-29 is recommended. The shear rate involved in determining viscosity by AS/NZS 2341.4 and AGPT/T111 must be calculated and recorded. AGPT/T111 has been retained in Table 8.1 to allow laboratories sufficient time to adopt AS/NZS 2341.4.

‘TBR’ throughout = to be reported.

Consistency 6% at 60 °C of S10E and S35E must be determined using mould B (breakpoint of 5 mm and a test speed of 1.5 mm/s). Other grades must be tested using mould A (breakpoint of 10 mm and a test speed of 1 mm/s).

‘NA’ throughout indicates that the property is considered not applicable for that PMB class.

Table 8.2:	Properties of Polymer Modified Binders for Asphalt Applications

| Test method | Class Binder property | A35P | A25E | A20E | A15E | A10E |
| --- | --- | --- | --- | --- | --- | --- |
| AS/NZS 2341.4 or AGPT/T111(1) | Viscosity at 165 °C (Pa.s) max.(1) | 0.6 | 0.6 | 0.6 | 0.9 | 1.1 |
| AGPT/T122 | Torsional recovery at 25 °C, 30 s (%) | 6–21 | 17–30 | 38–70 | 55–80 | 60–86 |
| AGPT/T131 | Softening point (°C) | 62–74 | 52–62 | 65–95 | 82–105 | 88–110 |
| AGPT/T125 | Stress ratio at 10 °C min. | TBR(2) | TBR | TBR | TBR | TBR |
| AGPT/T121 | Consistency 6% at 60 °C (Pa.s) min.(3) | 1000 | 400 | 500 | 900 | 1000 |
| AGPT/T121 | Stiffness at 25 °C (kPa) max. | 120 | 45 | 35 | 30 | 30 |
| AGPT/T108 | Segregation (%) max. | 8 | 8 | 8 | 8 | 8 |
| AGPT/T112 | Flash point (°C) min. | 250 | 250 | 250 | 250 | 250 |
| AGPT/T103 | Loss on heating (% mass) max. | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |

Notes

- L series Brookfield is recommended together with spindle SC4-31. The shear rate involved in determining viscosity by AS/NZS 2341.4 and AGPT/T111 must be calculated and recorded. AGPT/T111 has been retained in Table 8.2 to allow laboratories sufficient time to adopt AS/NZS 2341.4.

‘TBR’ throughout = to be reported.

Consistency 6% at 60 °C of A25E must be determined using mould B (breakpoint of 5 mm and a test speed of 1.5 mm/s). Other grades must be tested using mould A (breakpoint of 10 mm and a test speed of 1 mm/s).

Table 8.3:	Properties of Field-Produced Crumb Rubber Binders

| Property | Method | S15RF (1) | S18RF (1) | A27RF (2) |
| --- | --- | --- | --- | --- |
| Nominal rubber concentration (%) |  | 15 | 18 | 25–30 |
| Rubber content by analysis (%) min. | AGPT/T142(3) | 13 | 16 |  |
| Torsional recovery (%) min. | AGPT/T122 | 25 | 30 |  |
| Softening point (°C) min. | AGPT/T131 | 55 | 62 |  |
| Consistency 6% at 60 °C (Pa.s) | AGPT/T121 | Report | Report |  |

Notes:

- Specification for two grades of crumb rubber (see Table 8.4) available for either sealing class.

‘Dry mix’ asphalt is normally based on an asphalt mix design with the crumb rubber added at, typically, 25% crumb rubber in the total binder. Size 30 is normally used for the ‘Dry mix’ asphalt system.

A soxhlet extraction using toluene may also be used.

For sealing grades, the weekly sampling is from the mixing vessel after digestion but prior to the addition of cutter oil. Samples must be free of diluents for subsequent testing to be meaningful. The agreed digestion period (at mixing temperature) must be completed before sampling.

Crumb rubber must be:

- processed from waste tyres generated in Australia;

processed by a supplier accredited with Tyre Stewardship Australia or another organisation approved by the Principal; and

free from cord, wire, fluff and other deleterious material.

Table 8.4:	Properties of Crumb Rubber

| Test | Method | Size 16 | Size 30 |
| --- | --- | --- | --- |
| Grading | AGPT/T143 |  |  |
| passing 2.36 mm |  | 100 | 100 |
| passing 1.18 mm |  | 80 min. | 100 |
| passing 600 µm |  | 10 max. | 60 min. |
| passing 300 µm |  | – | 20 max. |
| Particle length (mm) max. | AGPT/T143 | 3 | 3 |
| Bulk density (kg/m3) | AGPT/T144 | Report | Report |
| Water content (%) max. | AGPT/T143 | 1 | 1 |
| Foreign materials – other than iron (%) max. | AGPT/T143 | 0.1 | 0.1 |
| Foreign materials – metallic iron (%) max. | AGPT/T143 | 0.1 | 0.1 |

# Frequency of Testing

The frequency of testing of PMBs and field produced crumb rubber binders must comply with the values specified in the following tables:

| Polymer Modified Binders: | Table 9.1 |
| --- | --- |
| Field-Produced Crumb Rubber Modified Binders: | Table 9.2 |

Table 9.1 Minimum Frequency of Testing of Polymer Modified Binders

| Binder property | Polymer Modified Binders for Sprayed Sealing Applications | Polymer Modified Binders for Asphalt Applications |
| --- | --- | --- |
| Viscosity at 165 °C (Pa.s) | Each batch | Each batch |
| Torsional recovery at 25 °C, 30 s (%) | Each batch | Each batch |
| Softening point (°C) | Each batch | Each batch |
| Stress ratio at 10 °C | Monthly | Monthly |
| Consistency 6% at 60 °C (Pa.s) | 3-monthly(1) | 3-monthly |
| Stiffness at 15 °C (kPa) or Stiffness at 25 °C (kPa) | 3-monthly(1) | 3-monthly |
| Compressive limit at 70 °C, 2 kg (mm) | 3-monthly | - |
| Segregation (%) | 3-monthly | 3-monthly |
| Flash point (°C) | Annually | Annually |
| Loss on heating (% mass) | Annually | Annually |

Notes:

- For classes S10E, S15E and S20E, the minimum testing frequency is 1-monthly.

Table 9.2 Minimum Frequency of Testing Field-Produced Crumb Rubber Binders

| Property | Minimum testing frequency |
| --- | --- |
| Rubber content by analysis (%) min. | Weekly(4) |
| Torsional recovery (%) min. | Weekly |
| Softening point (°C) min. | Weekly |
| Consistency 6% at 60 °C (Pa.s) | Weekly |

Annexure A: 	Summary of Hold Points, Witness Points and Records

The following is a summary of the Witness Points / Hold Points that apply to this specification and the Records that the Contractor must supply to the Principal to demonstrate compliance with this specification.

| Paragraph | Hold Point | Witness Point | Record |
| --- | --- | --- | --- |
| 17 a) |  |  | Test Results |
| 17 b) |  |  | Delivery docket |

Amendment Record

| Amendment no. | Clauses amended | Action | Date |
| --- | --- | --- | --- |
| - | New specification | New | January 2020 |

| Key |  |
| --- | --- |
| Format | Change in format |
| Substitution | Old clause removed and replaced with new clause |
| New | Insertion of new clause |
| Removed | Old clauses removed |